Laboratory Evaluation of a Non-Insecticide Multimodal Trap Using Carbon Dioxide, Lactic Acid, Ultraviolet-A Light, and Sound Frequency for Aedes aegypti Control

Article type : Original Article

Background: Dengue hemorrhagic fever remains an important public health problem in tropical and subtropical settings. Aedes aegypti is the primary dengue vector and is highly adapted to domestic environments. Because repeated chemical-based control may raise concerns regarding insecticide resistance and environmental exposure, practical non-insecticide tools are needed. This study evaluated a non-insecticide multimodal trap integrating carbon dioxide, lactic acid, ultraviolet-A light, and sound frequency for Aedes aegypti control.

Methods: This quasi-experimental study used a prototype development and laboratory evaluation approach. A total of 2,850 female Aedes aegypti mosquitoes were tested. The colony was established from eggs collected in Kendari City, Southeast Sulawesi, Indonesia. Female mosquitoes aged 3–5 days and maintained with sugar solution were used. The study included technical prototype assessment, single-attractant testing, multimodal combination testing in a glass chamber, and comparison with commercial mosquito traps in an enclosed room. Each treatment was performed in three replications. Data were analyzed using the General Linear Model and repeated ANOVA.

Results: Lactic acid was the most effective single attractant, trapping or killing 76%–80% of mosquitoes. The four-mode combination produced the highest effectiveness, with an average capture or mortality rate of 91.33% and a maximum rate of 98%. In the enclosed-room test, the multimodal trap performed better than the Mosquito Killer BG-360 and the negative control.

Conclusion: The non-insecticide multimodal trap demonstrated promising laboratory effectiveness against Aedes aegypti and may support adult mosquito control. Further semi-field and field trials are needed to evaluate effectiveness, feasibility, safety, and durability under real environmental conditions.

Keywords: Aedes aegypti; dengue vector; mosquito trap; multimodal attractant; vector control.

INTRODUCTION

Dengue hemorrhagic fever remains a serious public health problem in many tropical and subtropical regions, including Indonesia. This disease has a transmission pattern closely associated with environmental conditions, population density, human mobility, and the presence of disease vectors (1). Aedes aegypti is recognized as the primary vector of dengue hemorrhagic fever because of its high adaptability to residential environments, particularly clean water containers commonly found around households. The uncontrolled presence of adult mosquitoes and breeding sites can increase the risk of dengue transmission in the community. Therefore, vector population control remains an important strategy in dengue prevention, as transmission can be reduced when adult mosquito density and breeding habitats are consistently suppressed (2,3).

Efforts to control dengue vectors have been carried out through various approaches, including environmental, physical-mechanical, biological, and chemical control. Environmental control is implemented through mosquito breeding site eradication and the 3M Plus movement, such as draining water containers, covering water storage containers, recycling used items, and applying additional measures to prevent mosquito breeding. Physical-mechanical approaches include the use of mosquito nets, wire screens, electric rackets, and mosquito traps, while biological control can be conducted by utilizing larval predators such as larva-eating fish and Bacillus thuringiensis israelensis as larval control agents. These approaches are part of integrated vector management, which aims to suppress mosquito populations through a combination of safe and sustainable strategies (4–7).

Although these vector control approaches are widely applied, their implementation still faces several operational challenges. Source reduction and community-based mosquito control depend heavily on sustained household participation, regular monitoring, and environmental sanitation. Their effectiveness may decline when community involvement is inconsistent or when domestic environments provide numerous artificial containers suitable for mosquito breeding. These conditions indicate the need for additional tools that can complement existing vector control strategies and support adult mosquito reduction in domestic and peri-domestic settings (3,7).

The use of insecticides remains one of the most widely applied methods to suppress adult mosquito populations, especially during increases in dengue cases. Fogging is considered effective in rapidly reducing adult mosquito populations, although its effect is temporary if it is not accompanied by continuous source reduction (8). However, repeated and uncontrolled insecticide use may lead to reduced susceptibility of Aedes aegypti to certain active ingredients. Several studies in Indonesia have reported indications of resistance to pyrethroid and organophosphate insecticides. This condition shows that chemical-based control cannot be used as the sole option because it may reduce the long-term effectiveness of dengue vector control programs (9–11).

Innovation in mosquito traps has developed by utilizing host-seeking behavior. Female mosquitoes recognize the presence of hosts through a combination of olfactory, visual, thermal, and acoustic stimuli. Carbon dioxide does not only function as a chemical signal but can also strengthen host-seeking responses related to orientation toward heat or host body temperature (12). Carbon dioxide and lactic acid function as chemical cues, while ultraviolet-A light and sound frequency may serve as physical stimuli that influence mosquito flight orientation. Lactic acid is known to play an important role in increasing mosquito attraction to a stimulus source because this compound resembles a component of human body odor recognized by dengue vectors (13–17).

The multimodal approach offers an opportunity to improve trap effectiveness because several mosquito sensory systems are stimulated simultaneously. Traps that use only one attractant or limited stimuli tend to have less optimal attraction because they stimulate only one mosquito response pathway. Local studies show that differences in attractant types can produce variations in the number of trapped mosquitoes, while traps based on limited physical stimuli such as ultraviolet light and fans do not always demonstrate strong effectiveness. Therefore, the combination of several attractants and physical stimuli has the potential to strengthen mosquito orientation responses toward traps (18,19).

The combination of carbon dioxide, lactic acid, ultraviolet-A light, and sound frequency may provide more complex stimuli through the olfactory, visual, and auditory systems. Previous studies have shown that combined stimuli can increase landing and host-seeking responses compared with the use of a single attractant. This provides an important basis for developing mosquito traps based on multimodal combinations for adult mosquito control (20,21).

Based on this background, this study developed and tested a non-insecticide multimodal trap based on carbon dioxide, lactic acid, ultraviolet-A light, and sound frequency for Aedes aegypti control. This study aimed to determine the effectiveness of single attractants and multimodal attractant combinations on the number of dead or trapped mosquitoes and to compare the performance of the multimodal trap with commercial traps. The results are expected to contribute to the development of a practical and environmentally safer adult mosquito control tool for dengue vector management.

MATERIALS AND METHODS

Study Design

This study used a quasi-experimental design with a prototype development and laboratory testing approach. The research stages included identifying device requirements, designing the prototype, constructing the prototype, conducting technical component testing, testing attractants, evaluating multimodal combinations, and comparing the prototype with commercial mosquito traps. Testing was conducted using a glass chamber and an enclosed room to evaluate the trap’s ability to attract and kill mosquitoes.

Mosquito Samples

A total of 2,850 female Aedes aegypti mosquitoes were used in this study. The mosquito colony was established from eggs collected in Kendari City, Southeast Sulawesi, Indonesia. The eggs were reared under laboratory conditions until the adult stage. Female mosquitoes aged 3–5 days were selected for the experiments because this age range represents an active adult stage suitable for evaluating attraction and host-seeking responses. Before testing, the mosquitoes were maintained with sugar solution. Only active and morphologically normal female mosquitoes were included in the experiments.

Trap Prototype

The trap prototype consisted of a carbon dioxide-generating system, a lactic acid attractant container, an ultraviolet-A lamp, a sound device with a specific frequency, a suction fan, and an electric shock system. Technical testing was conducted to determine the working configuration of each component. Functional testing was then carried out in stages using single attractants, two-mode combinations, three-mode combinations, and four-mode combinations.

Experimental Procedures

The experimental procedures consisted of technical prototype testing, chemical attractant optimization, multimodal combination testing, and comparison with commercial mosquito traps. Each treatment was performed in three replications. In each test, female Aedes aegypti mosquitoes were released into the testing chamber or enclosed room, and the trap was operated according to the assigned treatment condition.

The tested stimuli included carbon dioxide, lactic acid, ultraviolet-A light, and sound frequency, either as single attractants or in multimodal combinations. The multimodal trap was compared with the Mosquito Killer BG-360, the Photocatalyst Mosquito & Fly Trap, and a negative control. Data were collected through direct observation of the number of mosquitoes killed or trapped at the 1st hour, 2nd hour, and 24th hour.

In this study, trap effectiveness was defined as the number of mosquitoes captured or killed by the trap during the observation period. Temperature, humidity, the number of mosquitoes in each test, observation time, and replication number were controlled to maintain comparability between treatments.

Statistical Analysis

The data were analyzed descriptively and inferentially. Inferential analysis was performed using the General Linear Model and repeated ANOVA, followed by post hoc pairwise comparisons to examine differences across observation times and treatments. Statistical significance was determined based on the p-value obtained from the analysis.

Ethical Consideration

This study used laboratory-reared Aedes aegypti mosquitoes and did not involve human participants, human biological samples, or vertebrate animals. Therefore, formal ethical approval was not required. All laboratory procedures were carried out with attention to biosafety, containment, and responsible handling of mosquito colonies.

RESULTS

Technical Performance of the Prototype

The technical test showed that the main components of the trap functioned according to the intended design. The configuration that produced the best mosquito response consisted of a carbon dioxide flow rate above 100 ml/minute, a 13-watt ultraviolet-A lamp, a sound frequency of 350–400 Hz, and a suction fan speed of 600 rpm. This configuration was used as the basis for testing attractant combinations because it induced more active mosquito behavior around the trap.

Table 1. Summary of the technical test results of the prototype

| Component | Best Configuration | Mosquito Response |
| --- | --- | --- |
| CO2 flow | >100 ml/ minute | Mosquitoes actively flew above the trap |
| UV-A lamp | 13 watts | Mosquitoes actively flew above the trap |
| Sound frequency | 350-400 Hz | Mosquitoes actively flew above the trap |
| Suction fan | 600 rpm | Mosquitoes were forcibly attracted while flying above the trap |

The technical test results showed that each component of the multimodal trap was able to generate active mosquito responses around the trap area. The best configuration of each component became the basis for using the device in the attractant and multimodal combination tests. These findings indicate that the prototype had fulfilled its initial technical functions before further effectiveness evaluation against Aedes aegypti was conducted.

Chemical Attractant Optimization

Chemical attractant optimization showed that 10% lactic acid produced the highest number of trapped mosquitoes, namely 40 mosquitoes or 80%. The carbon dioxide source tested in this study also produced the highest trapping response at the tested 10% formulation, with 39 mosquitoes or 78% trapped. These results were used as the basis for selecting the attractant condition in the multimodal combination test.

Table 2. Optimization results of lactic acid and carbon dioxide source

| Concentration (%) | Lactic acid: n (%) | Carbon dioxide source: n (%) |
| --- | --- | --- |
| 1 | 13 (26) | 7 (14) |
| 5 | 19 (38) | 21 (42) |
| 10 | 40 (80) | 39 (78) |
| 15 | 33 (66) | 8 (16) |

The optimization results showed that the 10% formulation was the most effective condition for both lactic acid and the carbon dioxide source used in this study. The decrease in the number of trapped mosquitoes at the 15% formulation indicates that increasing the attractant concentration does not always increase mosquito attraction. Therefore, the 10% formulation was selected as the basis for the multimodal combination test.

Effectiveness of Multimodal Combinations

Testing of various combinations showed that an increase in the number of attractant modes tended to increase the number of mosquitoes killed or trapped. The best single attractant was lactic acid, with a mean of 39.33 mosquitoes or 78.66%. The three-mode combination of ultraviolet-A light, sound frequency, and lactic acid reached 42 mosquitoes or 84%. The highest effectiveness was obtained from the four-mode combination of ultraviolet-A light, sound frequency, carbon dioxide, and lactic acid, with a mean of 45.66 mosquitoes or 91.33%.

Table 3. Summary of the Effectiveness of Attractants and Multimodal Combinations (n = 50)

| Attractant/Combination | Mean Number of Mosquitoes Killed/Trapped | Percentage (%) | Range (%) |
| --- | --- | --- | --- |
| UV-A | 28.67 | 57.34 | 52-64 |
| Lactic acid | 39.33 | 78.66 | 76-80 |
| CO2 | 32.33 | 64.66 | 54-78 |
| Sound frequency | 30.00 | 60.00 | 52-64 |
| UV-A + Lactic acid | 34.67 | 69.34 | 60-78 |
| UV-A + Sound frequency | 28.67 | 57.34 | 40-68 |
| UV-A + CO2 | 26.33 | 52.66 | 40-66 |
| Lactic acid + Sound frequency | 35.33 | 70.66 | 60-78 |
| Lactic acid + CO2 | 34.67 | 69.34 | 64-76 |
| Sound frequency + CO2 | 27.00 | 54.00 | 36-64 |
| UV-A + Sound frequency + CO2 | 34.67 | 69.34 | 66-76 |
| UV-A + Sound frequency + Lactic acid | 42.00 | 84.00 | 64-90 |
| UV-A + CO2 + Lactic acid | 37.66 | 75.32 | 62-90 |
| Sound frequency + CO2 + Lactic acid | 19.67 | 39.34 | 30-50 |
| UV-A + Sound frequency + CO2 + Lactic acid | 45.66 | 91.33 | 86-98 |

The test results showed that the four-mode combination produced the highest effectiveness compared with single attractants as well as two- and three-mode combinations. The increase in the number of mosquitoes killed or trapped indicates a synergistic effect among chemical, visual, and acoustic stimuli. This finding reinforces that the integration of carbon dioxide, lactic acid, ultraviolet-A light, and sound frequency has the potential to improve the performance of multimodal traps against Aedes aegypti.

Comparison with Commercial Traps

In the closed-room test, the multimodal trap demonstrated higher performance than the comparison traps. The difference between the multimodal trap and the Mosquito Killer BG-360, as well as the negative control, was statistically significant, while the difference with the Photocatalyst Mosquito & Fly Trap did not reach the 0.05 significance level. Nevertheless, the mean value of the multimodal trap remained higher, indicating its potential as a non-insecticide-based trapping technology.

Table 4. Comparison of the Multimodal Trap with Commercial Traps

| Comparison | Comparator Mean | Mean difference | p |
| --- | --- | --- | --- |
| Multimodal vs Mosquito Killer BG-360 | 12.33 | 22.33 | <0.001 |
| Multimodal vs Photocatalyst Mosquito & Fly Trap | 21.66 | 13.00 | 0.141 |
| Multimodal vs Negative Control | 1.33 | 33.33 | <0.001 |

The comparison results show that the multimodal trap performed better than the Mosquito Killer BG-360 and the negative control, with statistically significant differences. Although the difference with the Photocatalyst Mosquito & Fly Trap was not statistically significant, the mean value of the multimodal trap remained higher. These findings indicate that the integration of multiple attractants into a single trapping system has the potential to improve the effectiveness of Aedes aegypti control without relying on insecticides.

Statistical Analysis

The repeated ANOVA results showed a significant difference in the number of dead or trapped mosquitoes across the various attractant combinations (p < 0.001). The post hoc test across observation times also showed significant differences between the 1st hour, 2nd hour, and 24th hour. The number of mosquitoes that died or were trapped increased with longer exposure duration, particularly in the four-mode combination.

Table 5. Summary of Statistical Analysis Results

| Analysis | Comparison | p | Interpretation |
| --- | --- | --- | --- |
| Repeated ANOVA | Various attractant combinations | <0.001 | There was a significant difference among treatments |
| Post Hoc | 1st hour vs 2nd hour | 0.013 | Significantly different |
| Post Hoc | 1st hour vs 24th hour | <0.001 | Significantly different |
| Post Hoc | 2nd hour vs 24th hour | <0.001 | Significantly different |

The statistical analysis further confirmed that variations in attractant combinations had a significant effect on the number of mosquitoes that died or were trapped. Differences across observation times indicate that exposure duration also contributed to increased trap effectiveness. Therefore, both the attractant combination and exposure duration are important factors in the success of the multimodal trap against Aedes aegypti.

Discussion

The results of this study show that the non-insecticide multimodal trap was able to increase the effectiveness of capturing and killing Aedes aegypti. This finding is important for public health because Aedes aegypti is the primary vector of dengue transmission and has a high capacity to adapt to residential environments, particularly clean water containers commonly found around households. The relatively rapid life cycle of mosquitoes under optimal environmental conditions indicates that vector control must be carried out continuously and should not rely solely on a single intervention approach (2,22).

The effectiveness of the multimodal trap in this study can be explained through the host-seeking behavior of female mosquitoes. Aedes aegypti does not respond to only one stimulus, but instead uses a combination of chemical, visual, thermal, humidity, and acoustic cues to detect the presence of humans as a blood source. Carbon dioxide acts as a respiratory signal that can trigger host-seeking activity, while human body odor, including lactic acid, strengthens mosquito orientation toward the stimulus source. Understanding these sensory mechanisms provides the basis for developing attractant-based traps that mimic the presence of a human host (12,23,24).

Lactic acid was the most effective single attractant in this study. This result is consistent with the concept that lactic acid is one of the components of human body odor recognized by mosquitoes during host seeking. CO2 also makes an important contribution because it functions as a chemical cue indicating human presence through respiration. The combination of CO2 and lactic acid has the potential to strengthen the mosquito orientation response toward the trap, although the highest result in this study was only achieved when these two chemical attractants were combined with visual and acoustic stimuli (13,14,17,25).

Ultraviolet-A light and sound frequency function as physical stimuli that complement the activity of chemical attractants. Ultraviolet-A light assists the visual orientation of mosquitoes toward a light source, while sound frequencies within a certain range can influence flight activity and acoustic responses. When ultraviolet-A light and sound frequency are combined with carbon dioxide and lactic acid, the trap provides more complex stimulation, thereby increasing the likelihood that mosquitoes will approach the trap area. This condition enables the suction fan and electric shock system to work more effectively in capturing or killing mosquitoes (15,16,26).

The four-mode combination, consisting of ultraviolet-A light, sound frequency, carbon dioxide, and lactic acid, produced the highest effectiveness compared with single attractants and two- or three-mode combinations. This finding supports the concept of multimodal integration, in which mosquito responses to hosts are influenced by the simultaneous combination of multiple environmental cues. Traps that use only one attractant tend to have limited attractiveness because they stimulate only one sensory pathway. In contrast, multimodal traps stimulate the olfactory, visual, and auditory systems simultaneously, thereby producing a stronger response to the trap (20,21,27).

The results of this study also show that the multimodal trap performed better than the Mosquito Killer BG-360 and the negative control. This difference indicates that trap design, type of attractant, and the ability of the device to direct mosquitoes toward the capture system are key determinants of vector control effectiveness. Although the difference between the multimodal trap and the Photocatalyst Mosquito & Fly Trap was not statistically significant, the mean value of the multimodal trap remained higher. This finding suggests that the integration of multiple attractants into a single trap system may improve adult Aedes aegypti trapping performance. Previous evaluations of mosquito traps have also shown that trap success is influenced by device design, attractant composition, capture mechanism, and the ecological context in which the trap is applied (28–31).

The relevance of this study to vector-borne disease control lies in the potential contribution of a non-insecticide trap to adult Aedes aegypti management as part of integrated vector control. In dengue-endemic settings, vector control programs cannot rely solely on fogging or chemical insecticides, particularly when insecticide resistance and repeated chemical exposure are growing concerns. Several studies in Indonesia have reported reduced susceptibility or resistance of Aedes aegypti to commonly used insecticides, including pyrethroids and organophosphates. Therefore, a trap-based strategy using multimodal attractants may provide an additional tool for reducing adult Aedes aegypti populations in domestic and peri-domestic environments without directly increasing insecticide selection pressure (9–11).

The development of the multimodal trap is also consistent with the principles of integrated vector management, which emphasize the use of combined environmental, physical-mechanical, biological, and technological approaches for vector control. Source reduction remains important for dengue prevention, but its effectiveness depends greatly on sustained community participation, household monitoring, and environmental management. In this context, a non-insecticide multimodal trap may serve as a supporting technology that works in a more practical and measurable manner to reduce adult mosquito populations (3,6,7).

However, this study should be interpreted carefully. The testing was conducted only at laboratory and enclosed-room scales; therefore, the findings provide experimental evidence of trap effectiveness but do not directly demonstrate reductions in dengue incidence, adult mosquito density at the community level, or dengue transmission. Semi-field and field trials are needed to determine trap performance under real environmental conditions. In addition, component durability, energy consumption, user safety, ease of maintenance, production cost, and operational feasibility need to be evaluated so that this device can be developed as an applicable and sustainable vector control technology.

CONCLUSION

This study developed and evaluated a non-insecticide multimodal trap integrating carbon dioxide, lactic acid, ultraviolet-A light, sound frequency, a suction fan, and an electric shock system for Aedes aegypti control. Lactic acid at 10% and carbon dioxide at 10% were found to be the optimal concentrations in the attractant test. The four-mode combination of ultraviolet-A light, sound frequency, carbon dioxide, and lactic acid produced the highest effectiveness, with an average of 45.66 mosquitoes, or 91.33%, killed or trapped, and a maximum achievement of 98%. The multimodal trap was more effective than the Mosquito Killer BG-360 and the negative control. These findings suggest that the multimodal trap may serve as a supporting non-insecticide-based tool for adult Aedes aegypti vector control. Further research should include semi-field and field trials, design refinement, durability testing, safety assessment, and operational feasibility evaluation.

Acknowledgements

The authors would like to thank the laboratory staff and all supporting personnel who assisted during the mosquito rearing and laboratory testing procedures.

Funding

This research received no external funding.

Data Availability

The datasets generated and analyzed during the current study are available from the corresponding author upon reasonable request.

Ethical Approval

This study used laboratory-reared Aedes aegypti mosquitoes and did not involve human participants, human biological samples, identifiable personal data, or vertebrate animals. Therefore, formal ethical approval was not required. All laboratory procedures were conducted in accordance with biosafety, containment, and responsible mosquito-handling practices.

Consent to Participate

Not applicable. This study did not involve human participants.

Consent for Publication

Not applicable. This study did not involve identifiable personal data, clinical images, or individual participant information.

Conflict of Interest

The authors declare that there is no conflict of interest.

Authors’ Contributions

M contributed to the study conception, prototype development, data collection, data analysis, data interpretation, and manuscript drafting. TBTS contributed to study supervision, methodological refinement, data interpretation, and critical revision of the manuscript. All authors read and approved the final manuscript.

Declaration of Artificial Intelligence Use

Artificial intelligence tools were used only to assist with language editing and manuscript formatting. The authors reviewed, verified, and approved all content and remain fully responsible for the accuracy and integrity of the manuscript.

References

1.	Zaharudin Z, Subuh M, Khatab F. Analisis Lingkungan dan Kepadatan Penduduk terhadap Kejadian Demam Berdarah Dengue (DBD) di Daerah Pesisir Kabupaten Tanjung Jabung Barat. Jurnal Ilmu Kesehatan Masyarakat. 2024 Jan 24;13(06):397–406. doi:10.33221/JIKM.V13I06.3236

2.	Bhatt S, Gething PW, Brady OJ, Messina JP, Farlow AW, Moyes CL, et al. The global distribution and burden of dengue. Nature. 2013 Apr 25;496(7446):504–7. doi:10.1038/NATURE12060 PubMed PMID: 23563266.

3.	Widyantoro W, Nurjazuli N, Hanani Y. Pengendalian Demam Berdarah Dengue (DBD) Berbasis Masyarakat di Indonesia: Systematic Review. Jurnal Ilmu Kesehatan Masyarakat. 2021;10(03):191–9. doi:10.33221/JIKM.V10I03.1008

4.	Adrianto H, Silitonga HTH, Ritunga I, Santoso GA, Juwono MVC. Potensi pengendalian larva nyamuk Aedes aegypti (Linnaeus) dengan menggunakan tiga varietas  ikan cupang (Betta splendens): Potential for controlling Aedes aegypti (Linnaeus) mosquito larvae using three varieties of betta fish (Betta splendens). J Entomol Indones. 2024 Oct 1;21(2):130–9. doi:10.5994/JEI.21.2.130

5.	Sari E, Bahrina I, Bustanul S, Langsa U. Hubungan Pemberantasan Sarang Nyamuk (3M Plus) dengan Kejadian Demam Berdarah Dengue di Wilayah Kerja Puskesmas Karang Baru. Jurnal Promotif Preventif. 2025 Feb 6;8(1):80–5. doi:10.47650/JPP.V8I1.1665

6.	Go YF, Fifanto I, Yoga T, Murni M, Bate MY, Keken MNP, et al. Pemberantasan Sarang Nyamuk Melalui Sanitasi dan Pemanfaatan Tanaman Anti Nyamuk dengan Sistem Eco-Friendly Mosquito Control di Desa Pagersari, Ngantang, Jawa Timur. Jurnal Abdita Naturafarm. 2025;2(1):26–33. doi:10.70392/JAN.V2I1.2633

7.	Sutriyawan A, Darmawan W, Akbar H, Habibi J, Fibrianti F. Faktor yang Mempengaruhi Pemberantasan Sarang Nyamuk (PSN) Melalui 3M Plus dalam Upaya Pencegahan Demam Berdarah Dengue (DBD). Jurnal Ilmu Kesehatan Masyarakat. 2022 Jan 21;11(01):23–32. doi:10.33221/JIKM.V11I01.936

8.	Karo MB, Perangin-Angin S. Fogging dan Pemberantasan Sarang Nyamuk untuk Langkah Bersama Melawan Demam Berdarah di Wilayah Kerja Puskesmas Korpri Kecamatan Berastagi Kabupaten Karo Tahun 2024. BESIRU: Jurnal Pengabdian Masyarakat. 2024;1(12):1107–15. doi:10.62335/PZ6BWQ16

9.	Qibtiyah SM, Nuryady MohM, Susetyarini RrE, Permana TI, Sasongkojati DA. Analisis Status Resistensi Aedes aegypti terhadap Insektisida Cypermethrin 0,05% di Kecamatan Endemis Kabupaten Malang. Bioscientist : Jurnal Ilmiah Biologi. 2022 Jun 30;10(1):240. doi:10.33394/BIOSCIENTIST.V10I1.4988

10.	Silalahi CN, Tu WC, Chang NT, Singham GV, Ahmad I, Neoh KB. Insecticide Resistance Profiles and Synergism of Field Aedes aegypti from Indonesia. PLoS Negl Trop Dis. 2022;16(6):e0010501. doi:10.1371/JOURNAL.PNTD.0010501 PubMed PMID: 35666774.

11.	Triana D, Siregar FN, Utami EPWT, Suteky T, Wicaksono S. Entomological Parameters  And Characterization Of Insecticide Resistance In Dengue Vector Aedes Aegypti Larvae From Bengkulu City, Indonesia. Malaysian Journal of Public Health Medicine. 2021 Apr 24;21(1):96–102. doi:10.37268/MJPHM/VOL.21/NO.1/ART.467

12.	Reinhold JM, Chandrasegaran K, Oker H, Crespo JE, Vinauger C, Lahondère C. Species-Specificity in Thermopreference and CO2-Gated Heat-Seeking in Culex Mosquitoes. Insects. 2022 Jan 1;13(1):92. doi:10.3390/INSECTS13010092/S1

13.	Bello JE, Cardé RT. Compounds from human odor induce attraction and landing in female yellow fever mosquitoes (Aedes aegypti). Scientific Reports 2022 12:1. 2022 Sep 21;12(1):15638-. doi:10.1038/s41598-022-19254-w PubMed PMID: 36130984.

14.	Dormont L, Mulatier M, Carrasco D, Cohuet A. Mosquito Attractants. J Chem Ecol. 2021 May 1;47(4–5):351–93. doi:10.1007/S10886-021-01261-2 PubMed PMID: 33725235.

15.	Nakata T, Simões P, Walker SM, Russell IJ, Bomphrey RJ. Auditory sensory range of male mosquitoes for the detection of female flight sound. J R Soc Interface. 2022 Aug 24;19(193):20220285. doi:10.1098/RSIF.2022.0285 PubMed PMID: 36000227.

16.	Peach DAH, Ko E, Blake AJ, Gries G. Ultraviolet inflorescence cues enhance attractiveness of inflorescence odour to Culex pipiens mosquitoes. PLoS One. 2019 Jun 1;14(6):e0217484. doi:10.1371/JOURNAL.PONE.0217484 PubMed PMID: 31163041.

17.	Mubarak, Baskoro T, Satoto T, Hartini S, Tontowi A. Systematic review: Effectiveness of combination of lactic acid attractants for control of dengue vector Aedes spp. J Vector Borne Dis. 2021 Jun 1;58(2):99–105. doi:10.4103/0972-9062.316276 PubMed PMID: 35074942.

18.	Rahayu ER, Atikah D, Setiyadi A, Sulistyo A, Putri ST, Purba S, et al. Pemanfaatan Modifikasi Mosquito Killer Sebagai Alternatif Pengendalian Nyamuk Aedes Aegypti: Utilization of Modified Mosquito Killer as an Alternative for Aedes Aegypti Mosquito Control. Journal of Health. 2023 Dec 11;10(1):073–8. doi:10.30590/JOH.V10N1.530

19.	Saputra R, Gemala M, Oktarizal H. Pemanfaatan Fermentasi Singkong, Air Tebu, Gula Merah dan Kulit Pisang dalam Optimalisasi Perangkap Nyamuk (Trapping) sebagai Atraktan Perangkap. JURNAL KESEHATAN PERINTIS. 2022 Jun 30;9(1):23–30. doi:10.33653/JKP.V9I1.782

20.	Carnaghi M, Belmain SR, Hopkins RJ, Hawkes FM. Multimodal synergisms in host stimuli drive landing response in malaria mosquitoes. Scientific Reports 2021 11:1. 2021 Apr 1;11(1):7379-. doi:10.1038/s41598-021-86772-4 PubMed PMID: 33795798.

21.	McMeniman CJ, Corfas RA, Matthews BJ, Ritchie SA, Vosshall LB. Multimodal integration of carbon dioxide and other sensory cues drives mosquito attraction to humans. Cell. 2014 Feb 27;156(5):1060–71. doi:10.1016/j.cell.2013.12.044 PubMed PMID: 24581501.

22.	Lema YNP, Almet J, Wuri DA. Gambaran Siklus Hidup Nyamuk Aedes sp.  Di Kota Kupang. Jurnal Veteriner Nusantara. 2021 Dec 19;4(1):2–2. doi:10.35508/JVN.V4I1.6030

23.	Coutinho-Abreu I V., Riffell JA, Akbari OS. Human attractive cues and mosquito host-seeking behavior. Trends Parasitol. 2022 Mar 1;38(3):246–64. doi:10.1016/J.PT.2021.09.012 PubMed PMID: 34674963.

24.	Yan J, Gangoso L, Ruiz S, Soriguer R, Figuerola J, Martínez-de la Puente J. Understanding host utilization by mosquitoes: determinants, challenges and future directions. Biol Rev Camb Philos Soc. 2021 Aug 1;96(4):1367–85. doi:10.1111/BRV.12706 PubMed PMID: 33686781.

25.	Wu Y, Wang J, Li T, Liu Q, Gong Z, Hou J. Effect of different carbon dioxide (CO2) flows on trapping Aedes albopictus with BG traps in the field in Zhejiang Province, China. PLoS One. 2020 Dec 1;15(12):e0243061. doi:10.1371/JOURNAL.PONE.0243061 PubMed PMID: 33259534.

26.	Wolff GH, Lahondère C, Vinauger C, Rylance E, Riffell JA. Neuromodulation and differential learning across mosquito species. Proc Biol Sci. 2023 Jan 11;290(1990). doi:10.1098/RSPB.2022.2118 PubMed PMID: 36629098.

27.	Martinez J, Showering A, Oke C, Jones RT, Logan JG. Differential attraction in mosquito-human interactions and implications for disease control. Philos Trans R Soc Lond B Biol Sci. 2021 Feb 15;376(1818). doi:10.1098/RSTB.2019.0811 PubMed PMID: 33357061.

28.	Barrera R. New tools for Aedes control: mass trapping. Curr Opin Insect Sci. 2022 Aug 1;52. doi:10.1016/j.cois.2022.100942 PubMed PMID: 35667560.

29.	Jaffal A, Fite J, Baldet T, Delaunay P, Jourdain F, Mora-Castillo R, et al. Current evidences of the efficacy of mosquito mass-trapping interventions to reduce Aedes aegypti and Aedes albopictus populations and Aedes-borne virus transmission. PLoS Negl Trop Dis. 2023;17(3):e0011153. doi:10.1371/JOURNAL.PNTD.0011153 PubMed PMID: 36877728.

30.	Paiz-Moscoso KE, Cisneros-Vázquez LA, Danís-Lozano R, Rodríguez-Rojas JJ, Rebollar-Téllez EA, Sánchez-Casas RM, et al. Design and Evaluation of a Sticky Attractant Trap for Intra-Domiciliary Surveillance of Aedes aegypti Populations in Mexico. Insects. 2023 Dec 1;14(12). doi:10.3390/INSECTS14120940 PubMed PMID: 38132613.

31.	Tambwe MM, Saddler A, Kibondo UA, Mashauri R, Kreppel KS, Govella NJ, et al. Semi-field evaluation of the exposure-free mosquito electrocuting trap and BG-Sentinel trap as an alternative to the human landing catch for measuring the efficacy of transfluthrin emanators against Aedes aegypti. Parasit Vectors. 2021 Dec 1;14(1). doi:10.1186/S13071-021-04754-X PubMed PMID: 34016149.